# Enterprise-level Multifunctional Fluid Control Gateway

manual of operation

V1.1

April 20,2023

catalogue

Enterprise-level Multi-functional Fluid Control Gateway 1	1

1. Product Overview 4	4

2. Log in to the WEB Interface 4	4

3. Be familiar with all the functions of the web page 4	4

3.1. System Status 4	4

Device Information 5	5

Interface Status 5	5

Internal network IP traffic: 6	6

Application Traffic 7	7

3.2. Network Configuration 7	7

Interface Configuration – External Network Configuration 8	8

Interface Configuration-Internal Network/DHCP 8	8

Interface Configuration-Physical Port Division 10	10

Routing Rules – Multi-Line Traffic Distribution Rule 11	11

Routing Rules – Static Routing 12	12

Dynamic Domain 12	13

NAT/Port Mapping 13	13

3.3. Epidemic Control Strategies 14	15

Intelligent Flow Control 14	15

Policy Speed Limit 14	15

Free Flow Control 15	16

3.4. AC Management 15	16

AP Device List 15	16

AP Configuration Template 17	18

AP Upgrade 18	19

Wireless Roaming 18	19

Automatic Channel 19	20

Audit Configuration and Server Location 20	20

3.5. Certified Access 21	21

Authentication Configuration 21	21

PPoS 21	22

Portal Authentication 22	23

Radius Billing 23	23

Authenticated User 23	24

authenticated user status 25	26

Department/Level Definition 25	26

3.6. Behavioral Control 26	27

Apply Firewall 26	27

URL Redirect 26	27

Domain Redirect 27	28

3.7. Object Management 28	29

Basic Object – Time Object 28	29

Basic Object – Source IP Object 28	29

Basic Object – Port Object 29	30

Basic Object – Destination IP Object 29	30

Application Object – Built-in Application Object 30	31

Application Object – Custom Application Object 31	32

3.8. Safety Protection 32	34

IP-MAC Binding 32	34

Connection Limit: 32	34

Internal Network Abnormality Detection 32	34

Internal Network Attack Protection 34	36

Ping access blocked to external network / External network login 34	36

3.9. Log Recording 35	37

User Authentication Log 35	37

Online User Count Log 35	37

Interface Traffic Log 36	38

System Log 36	38

3.10. VPN Applications 36	38

PPTP	36	38

L2TP	38	40

VTUNS	38	40

3.11. Equipment Maintenance 40	42

Device Upgrade 40	42

Password Change 40	42

Ping Test 41	43

Configuration File Maintenance 41	43

Reboot Device 43	45

Scheduled Task 43	45

Time Synchronization 44	46

Cloud Platform Configuration 44	46

- Product Overview

This product is a multifunctional flow-control gateway wireless AC unit featuring multi-line diversion and load balancing capabilities, delivering high-capacity, high-performance, highly reliable wireless data control services with easy installation and maintenance. It offers flexible networking and energy-efficient advantages.

- Log in to the Web Interface

  - After the gateway is powered on and starts up normally, the Run status indicator on the device will flash regularly. Connect the network cable to the LAN port to ensure proper network connectivity between the logged-in client devices and the gateway.

  - Enter 172.16.0.1:2011 in the browser; the default username and password are admin

After successful login, go to the homepage

Figure 1 – Homepage

- Familiar with all features of web pages

  - system mode

Displays comprehensive gateway information, including interface statuses, internal network client details, uplink and downlink speeds, real-time uplink and downlink speeds for each application, and total traffic.

facility information

Device Information: Displays comprehensive gateway information

Number 1: Shows the physical connection status of the interface; colored icons indicate a connected connection

Number 2: IP address corresponding to the display interface

Number 3: Unique device ID for remote connection

Number 4: Number of terminals currently connected to the gateway, excluding APs.

Number 5: Display Device Model

Click on online users to filter APs and devices by user, IP, or MAC address, and view corresponding IP and MAC addresses.

Interface State

View comprehensive information for each interface. View interface details and WAN port speed measurements.

Internal Network IP Traffic

You can view traffic information for each internal network terminal's independent usage, as well as the link tracking table.

View this user's data type, speed, and number of connection protocols

Application Traffic

View the proportion of downloads and uploads for each traffic type.

  - line configuration

Configuration information for setting up external and internal networks, including internal network routing

Interface Configuration – External Network Configuration

Internet connection method – Select the appropriate method based on your needs

- ADSL/PPPOE: Enter your broadband account and password

- Fixed IP: Enter the IP address, mask, gateway, and DNS provided by your carrier

- DHCP: Connect directly to the provider's network to obtain an IP address

Line interruption detection – Ping testing (114DNS, Tencent Official Website, Alibaba DNS, Baidu DNS): If consecutive pings fail, high latency occurs, and no data exchange is detected, the connection is flagged as abnormal. For poor-quality connections, dial-up attempts will be repeated, DHCP configurations reconfigured, and fixed IPs handled offline. Offline connections are excluded from load balancing. In multi-line environments, line interruption detection should be enabled to ensure automatic switching only when specific lines become offline.

Note: Dedicated network lines in Chengdu and Chongqing cannot be selected (cannot be tested); for certain dedicated lines, line testing cannot be enabled if the operator prohibits ping requests.

PING IP for detection: Default is 0.0.0.0, indicating use of built-in IPs (114DNS, Tencent official website, Alibaba DNS, Baidu DNS) for testing. If your local DNS can be pinged or another public IP with lower latency is available, enter it as the detection reference.

Note: This recommendation should be evaluated by a qualified technician before entering the test IP address.

Interface Configuration – Internal Network/DHCP

Number 1: LAN port address. By default, ports 1–4 belong to the same LAN and are used for accessing the AC.

Number 2: Configuration information for the IP segment allocated from the IP address pool; it must not match the IP segment obtained from the WAN port.

Number 3: DHCP static allocation, allowing managed binding of IP and MAC addresses.

View DHCP allocation status records

Note: If AC is connected via a bypass router, check "Assign IP only to AP".

Interface Configuration – Physical Port Allocation

Used to divide multiple WAN ports and LAN ports according to requirements

Number 1: If you need to divide multiple LANs, uncheck this option; LANs 1–4 will have different addresses

Number 2: Enable hardware NAT forwarding to route data directly without passing through the CPU, improving network speed.

Interface Configuration – Sub-interface Configuration

Created for

Routing Rules – Multi-path Traffic Distribution Rules

A single line does not require traffic distribution rules. When two or more WAN ports connect to the external network, you can configure different source addresses and traffic distribution modes, which are divided into three types:

- Session load balancing: Distributes all internet connections across multiple lines. For example, when Zhang San uses Xunlei for downloads, the software simultaneously sends numerous links, which are distributed across three lines. Each line generates traffic that returns to Xunlei, achieving bandwidth synergy.

- Source-plusDestination Address Traffic Distribution: Building upon session-based traffic distribution, the system identifies source and destination addresses to route traffic to respective network paths. For example, assume Zhang San simultaneously visits three websites—Industrial and Commercial Bank of China, JD.com, and Taobao—with their IP addresses denoted as A, B, and C respectively. Using this source-plusdestination address approach, all traffic is distributed across three external network paths. The final outcome is that Zhang San's requests to Industrial and Commercial Bank of China are routed exclusively through Path 1; those to JD.com through Path 2; and those to Taobao through Path 3.

- IP Traffic Distribution: Traffic is consistently routed through a single line based on source address. Taking an environment with three external networks serving nine users as an example, all users are assigned to these three networks respectively: Zhang San uses Line 1 exclusively; Li Si uses Line 2 exclusively; Wang Wu uses Line 3 exclusively; Zhao Liu uses Line 1 exclusively—each user is assigned to one fixed line accordingly. Since each user is dedicated to a specific line, their internet speed is limited by that line's bandwidth capacity.

Weight: Weight refers to a "proportion" and is only applicable for IP traffic distribution. Consider an environment with three external networks serving 12 users: all users are routed to these networks, with Line 1 assigned weight 3, Line 2 weight 2, and Line 3 weight 1. The distribution results are as follows: Line 1 handles 6 users, Line 2 handles 4 users, and Line 3 handles 2 users.

(Weight represents a ratio; for example, three lines with weights of 4, 2, and 1 result in a ratio of 4:2:1.)

Summary: IP traffic distribution is particularly suitable for networks with numerous connections, minimizing simultaneous IP usage by multiple users at WAN ports and enhancing overall IP utilization. It is widely adopted in community broadband networks serving e-commerce platforms like Taobao and Amazon, as well as in gaming studios—where excessive access from a single WAN port may be flagged by platforms as automated ordering or idle activity.

Session load balancing is suitable for highly demanding multi-threaded download scenarios, such as P2P downloads and game update servers.

Source and destination address routing enhances compatibility based on session routing, with default recommendation enabled

Routing Rules – Static Routing

When a dedicated network is specified, you need to configure the terminal to forward traffic to the corresponding gateway when accessing the designated IP range.

View Static Routing Table

Dynamic Domain

Route management is performed from the external network by accessing the dynamic domain name, which is primarily provided by the dynamic domain service provider.

remarks ：

1. The router only handles IP reporting; the accuracy and speed of resolution depend on the 3322 service provider.

2. Some carriers assign internet IPs as local area network (LAN) addresses, such as 10.10.99.99, which are inaccessible from the external network. Configuring dynamic domain names on such LAN IPs is ineffective.

NAT/ port mapped

Used to map LAN ports to the public network

DMZ main engine

This feature addresses the issue where external networks cannot access internal network servers after installing a firewall. Click to enable the DMZ host and manually enter the address and external port to activate this function.

Source Address Conversion | Destination Address Conversion								Source Address Conversion | Destination Address Conversion

  - Flow Control Strategy

Manage the network speed of terminals by evenly distributing bandwidth or limiting terminal bandwidth.

Intelligent Flow Control

For example, a dial-up fiber with an upstream speed of 20 Mbit/s and a downstream speed of 100 Mbit/s can be configured with an upstream speed of 2000 KB/s and a downstream speed of 10000 KB/s. Configuring line bandwidth is crucial; Intelligent Flow Control automatically limits speeds based on the assigned bandwidth (the bandwidth configuration takes effect only if you check the "Enable Intelligent Flow Control" option).

Policy-based Speed Limit

Apply speed limits based on different source address rules

Free-flow control

Do not be constrained by the overall network speed control of intelligent flow management; instead, set a separate policy to limit its maximum bandwidth.

  - AC manage

AP Device List

Show all APs connected to the AC gateway for quick viewing and management.

Number 1: View the number of deployed APs. Green indicates deployed APs, while red shows the total number of connected APs.

Number 2: Filter to display only online or offline APs.

Number 3: Filter to display only APs with a single model.

Number 4: You can search based on the AP device's IP address, MAC address, name, model, or version number.

No.5: WIFI Analyzer, used to scan all WIFI channels on this AP's 2.4 GHz or 5.8 GHz frequency bands.

Number 6: Edit parameters and configurations for a single AP.

Number 7: Displays the SSID (wireless WiFi name) and channel of the AP's 2.4G,5.8G, and 5.8G2 bands. Tap the green icon to connect a device to this AP.

Number 8: AP server login address.

Click on the AP list to edit individual AP configurations on the right side

Main Editing Sections:

Number 1: Select parameters to edit for the current frequency band.

Number 2: Edit the WIFI name issued by AP.

Number 3: Encryption method – typically WPA2 PSK.

Number 4: Edit your WIFI password. Click the lock button on the right to view it.

Number 5: Select a protocol mode with a different current frequency band.

Secondary Sections:

Device Name, Device Note: Used to distinguish all locations of this AP; the note can typically be set as installation location or coverage location.

timed restart: Set this AP to automatically restart at a specific time each day or every 1–10 days.

AP Management Password: The login password for this AP's management page.

Wireless Status: Select this frequency band to enable or disable it.

Channel: You can choose automatic channel selection, where the AP will automatically find the optimal channel, or manually select a specific channel.

Broadcast SSID: Activated, the device can be searched; disabled, the SSID is hidden.

User isolation: Controls whether terminals under this AP can access each other.

Transmission Power: Default is 100%; available options include 75%,50%,25%, and 12%.

AP Coverage Threshold: If the detected terminal's connection strength falls below the threshold, the AP will exclude it.

VLAN ID: Sets the ID for the AP wireless service VLAN.

Number of connected users: Maximum number of devices connected to this AP.

Virtual Wi-Fi: Create up to 3 virtual Wi-Fi networks per frequency band, each with a unique SSID and password.

AP Configuration Template

Select a model to add a template; each model corresponds to one template. Configure the SSID and password, then select all APs of the same model in the AP list and choose the corresponding template from Application Configuration.

AP upgrade

You can choose online upgrade or upload firmware for local upgrade.

Seamless roaming

KVR Switching Threshold Configuration

Automatic Channel

Configure the timing for re-scheduling and re-channeling coordination

Channel table: View the channels and signal strength of other Wi-Fi networks for each AP.

Audit Configuration and Server Location

For custom development integration with the audit system

  - Certified Internet Access

Typically, internet users can obtain an IP address by configuring the network card's IP address or letting the router assign one via DHCP, allowing direct internet access.

authenticated internet access means that a user identity is required to access the internet.

Authentication Configuration

Control the authentication switch for the corresponding LNA port.

Note: Activating any of these switches blocks traffic on LAN 2. Only authenticated users are allowed to access the internet.

PPPoE attestation

PPPoE authentication – Used for community broadband, where internal network users access the internet via PPPoE dial-up. The dial-up credentials (username and password) are configured on the router (if connected to a Radius billing system, they must be set up in that system).

For address pool allocation, it is recommended to use a LAN address that does not belong to the same subnet as the LAN port's IP address. For example, if the LAN port's IP address is 192.168.1.1, the address pool cannot be 192.168.1.xxx.

DNS recommends using the local carrier's DNS.

Portal attestation

One-click authentication: Used in hotels and guesthouses to prevent camera holes from connecting to WiFi. Requires an additional manual click step, equivalent to self-service approval.

WEB Password Authentication: Users connected to the router (e.g., smartphones) log in by entering their username and password in the pop-up authentication window. The WEB password account credentials are created on the router (if integrated with Radius billing, creation is required in the Radius billing system).

SMS Authentication (Paid): Enter your phone number to receive a random code from the SMS platform for online authentication.

Authentication Page Customization: Customize upload image assets

Radius charging

Authenticating User

The figure below shows four types of users.

Authenticate User Status

You can see active users; the green icons in the image represent them.

Department/Level Definition

Department Division: Manages departments and levels to associate internet users

Department Classification Purpose: Users must be assigned to a specific department; therefore, departments need to be defined before creating users. For companies, departments are typically organized by administrative divisions (e.g., Sales Department, Production Department). For residential communities, departments are usually established based on user locations (e.g., Building 35 East, Building 88 West). Once departments are defined, users can be subject to various control rules corresponding to their assigned department.

Purpose of Level Classification: Users must select their appropriate level; therefore, levels need to be defined before creating users. For organizations, levels are typically assigned based on user permissions and roles (e.g., senior executives, regular employees). For residential communities, levels are usually determined by the purchased bandwidth (e.g., 2M,4M). Once levels are defined, various control measures or speed-limiting rules can be applied accordingly.

  - behavior control

Apply Firewall

Configure the target IP, port, and application for permitted or directly blocked access based on source and time settings.

URL Redirection

Set to automatically redirect to the destination URL when the terminal accesses the original URL (effective after clearing browser cache).

Domain Redirection

Set the terminal to automatically resolve domain names to specified IPs when accessing them (effective after clearing browser cache)

  - Object Management

Basic Object – Time Object

Basic Object – Source IP Object

Basic Object – Port Object

Basic Object – Destination IP Object

Application Object – Built-in Application Object

Application Object – Custom Application Object

Apply based on domain definition

Define applications by destination IP and port

  - safe guarding

IP-MAC binding

After binding IP-MAC, the inability to arbitrarily modify IP addresses prevents IP conflicts that could disrupt other users' normal internet access.

Connection Count Control

Limit the maximum number of TCP and UDP connections for the source object

Internal Network Abnormality Detection

Enabling DHCP detection allows you to check for other DHCP servers on the local network that may cause IP address conflicts.

It is normal for an AC gateway to detect the primary router as its own when acting as a secondary router.

Enabling loop detection helps identify whether switches in a local network have link loops, which can cause broadcast storms and result in slow, unstable internet performance.

Common examples:

A private third-party router is connected to the LAN port, resulting in a DHCP conflict when improperly connected to the router's LAN port.

Internal Network Attack Protection

Ping is prohibited on the external network / Login to the external network

View remote login parameters and configurations

  - Log Record

User Authentication Log

Check device online and offline status

Online User Count Log

Add the endpoint as a user object for recording

Interface Traffic Log

system log

  - VPN apply

PPTP

To use this feature, the AC must serve as the primary router, and its WAN port must be connected to a public IP address provided by the carrier.

The gateway IP and address pool should be configured according to actual DNS requirements.

Click to authenticate users and proceed to create a VPN authenticated user

L2TP

To use this feature, the AC must serve as the primary router, and its WAN port must be connected to a public IP address provided by the carrier.

The gateway IP and address pool should be configured according to actual DNS requirements.

VTUNS

The network establishes a virtual channel over TCP/IP between two local area networks for integration. The higher-performance network segment serves as the server, with both segments having distinct IP addresses; the server must possess a public IP address.

Tunnel management: Add custom tunnel names and IDs, and specify the internal network segment for the VPN client (e.g., 192.168.1.0 with mask 255.255.255.0). Note that tunnel names and IDs must match exactly between the server and client configurations.

  - plant maintenance

Device Upgrade

Upgrade online or select a specific firmware upgrade

Change Password

Ping check

Check whether a path exists between AC and the specified IP.

Multiple Ping

Configuration File Maintenance

Used to export and import gateway configuration information, and restore it to factory settings

Mail Backup: Enter the sender and recipient email addresses to periodically collect backup files

Reboot the device

timed task

Set up the gateway's scheduled operation

time lock

You can select different time zones and primary time servers

Cloud Platform Configuration

AC is bound to the cloud platform, allowing remote management of both AC and AP.

Enter the cloud platform address, register an account on the platform, and after logging in, enter the AC's LAN port MAC address to bind it.