LBTD216G

16-port 10/100/1000BASE-TX +2G SFP Industrial Ethernet Switch

Interface

- 16-Port 10/100/1000Base-Tx RJ-45 with auto  negotiation and auto-MDI/MDI-X function

- 2-Port SFP uplink

Industrial conformance

- 12V to 52V DC, redundant power input

- -40℃ to 80℃ operating temperature

- IP40 aluminum housing

- Supports Air discharge: ±12kV

- Contact discharge: ±8kV

- Free Fall, Shock and Vibration Stability

- DIN-rail hardware design

Slim Type with more Practicability and Convenience

LBTD216G an hardened 16-port Industrial Ethernet Switch, provides non-blocking wire-speed performance in harsh industrial environment. It provides 16-port 10/100/1000BASE-TX RJ45 copper interfaces +2 Port 10/100/1000 SFP delivered in an IP40 rugged, strong case with redundant power system. Its slim case is suitable for such applications as surveillance system deployments, control management and wireless service in climatically demanding environments with wide temperature range from -40℃ to 80℃.

The LBTD216G series provides the flexibility to all kinds of 10/100Mbps Ethernet Media on RJ-45 port and performs highly stable fiber performance.

The LBTD216G series is packaged in a compact IP40 case that allows either DIN rail or panel mounting to have efficient usage of cabinet space. It provides an integrated power supply with a wide range of voltages for worldwide operation. It also offers dual-redundant, reversible polarity 12V DC to 52V DC power supply inputs for high availability applications requiring dual or backup power inputs.

Specifications:

| Model No. | LBTD216G |
| --- | --- |
| 10/100/1000BASE-TX Port | 16 port RJ-45 auto-MDI / MDI-X 2 port SFP |
| Performance Specification | Bandwidth: 56Gbps Packet Buffer Memory:4.1Mbit Packet Forwarding Rate: 42Mpps MAC Address Table: 8K |
| Installation | DIN rail |
| Maximum Frame Size | 9000bytes packet size |
| Flow Control | Back pressure for half duplex, IEEE 802.3x pause frame for full duplex |
| Enclosure | IP40 aluminum housing |
| LED Indicator | Power: Red Ethernet: Yellow POE: Green P.S LBTD216G not with PoE function |
| Power Input | 12 to 52V DC redundant power |
| Power Consumption | 10 watts |
| Surge protection | ±4KV |
| Network Protocols | IEEE802.3 10BASE-T; IEEE802.3i 10Base-T; IEEE802.3u;100Base-TX/FX; IEEE802.3ab 1000Base-T; IEEE802.3z 1000Base-X; IEEE802.3x; |
| Network cables | 10BASE-T: Cat3,4,5 UTP(≤100 meter) 100BASE-TX: Cat5 or later UTP(≤10meter) 1000BASE-TX: Cat6 or later UTP(≤100 meter) |
| Industry Standard | IEC 61000-4-4 (EFT), Level 3 IEC 61000-4-5 (Surge), Level 3 Shock: IEC 60068-2-27 Free fall: IEC 60068-2-32 Vibration: IEC 60068-2-6 |
| MTBF | >300,000hours |
| Certification | CE FCC Rohs compliance |
| Dimensions (W x D x H) | 175.6 x 135 x 45.5 mm |
| Weight | Product Weight: 0.9KGS Packing Weight: 1.0KGS |
| Working Environment | Working temperature: -40～80℃ ; Storage temperature: -40～80℃ Relative Humidity: 5%~95 %( no condensation) |
| Warranty | 1-year replacement with new item; 3-years for main parts. |

Dimensions