LBT128BGS-SFP

8-port 10//100/1000BASE-TX+12G SFP Managed Industrial Switch

Features

- Data control: support 802.3x full duplex flow control, support network storm suppression

- Redundant network: support STP/RSTP/MSTP, support ERPS (self-healing time <20ms)

- Support IPv6, static routing

- Support DHCP Server, DHCP replay, Qos，IGMP Snooping

- Support port-based VLAN Port, IEEE 802.1Q VLAN and GVRP protocol, simple network planning

- QoS: Support QoS (IEEE 802.1p/1Q) and TOS/ Diffserv to increase network stability

- Security management: support ACL access control list, support 802.1x, support user classification management

- Management function: support Web, SNMP management mode

- Monitoring maintenance: support port mirroring, interface status monitoring, log management

- Support 1588 clock protocol

LBT128BGS-SFP an hardened 20-port Industrial Ethernet Switch, provides non-blocking wire-speed performance in harsh industrial environment. It provides 8-port 10/100/1000BASE-TX RJ45 +12G SFP delivered in an IP40 rugged, strong case with redundant power system. Its slim case is suitable for such applications as surveillance system deployments, control management and wireless service in climatically demanding environments with wide temperature range from -40℃ to 80℃.

The LBT128BGS-SFP provides the flexibility to all kinds of 10//100/1000Mbps Ethernet Media on RJ-45 port and performs highly stable fiber performance.

The LBT128BGS-SFP is packaged in a compact IP40 case that allows either DIN rail or panel mounting to have efficient usage of cabinet space. It provides an integrated power supply with a wide range of voltages for worldwide operation. It also offers dual-redundant, reversible polarity 12V DC to 52V DC power supply inputs for high availability applications requiring dual or backup power inputs.

ROBOT

ROBOT

Specifications:

| Model No. | LBT128BGS-SFP |
| --- | --- |
| 1000Base-TX Port | 8 port RJ-45 auto-MDI / MDI-X |
| 1000Base-FX Port | 12G SFP Slot |
| Performance Specification | Bandwidth(switch capacity): 106Gbps Packet Buffer Memory:16Mbit Packet Forwarding Rate: 78Mpps MAC Address Table: 8K |
| Installation | DIN rail |
| Maximum Frame Size | 9600bytes |
| Flow Control | Back pressure for half duplex, IEEE 802.3x pause frame for full duplex |
| Enclosure | IP40 aluminum housing |
| LED Indicator | Power: Red Fiber: Link 2(Green) Ethernet: Yellow |
| Power Input | 12 to 52V DC redundant power |
| Power Consumption | <10 watts |
| Surge protection | ± 4KV |
| Network Protocols | IEEE802.3 10BASE-T; IEEE802.3i 10Base-T; IEEE802.3u;100Base-TX/FX; IEEE802.3ab 1000Base-T; IEEE802.3z 1000Base-X; IEEE802.3x; |
| Network cables | 10BASE-T: Cat3,4,5 UTP(≤100 meter) 100BASE-TX: Cat5 or later UTP(≤10meter) 1000BASE-TX: Cat6 or later UTP(≤100 meter) |
| Industry Standard | FCC CFR47 Part 15,EN55022/CISPR22, Class A IEC61000-4-2 (ESD): ±8kV (contact), ±12kV (air) IEC61000-4-3 (RS): 10V/m (80~1000MHz) IEC61000-4-4 (EFT): Power Port: ±4kV; Data Port: ±2kV IEC61000-4-5 (Surge): Power Port: ±2kV/DM, ±4kV/CM; Data Port: ±2kV IEC61000-4-6 (CS): 3V (10kHz-150kHz); 10V (150kHz-80MHz) IEC61000-4-16 (Common mode conduction): 30V (cont.), 300V (1s) |
| Certification | CE FCC Rohs compliance |
| MTBF | >300,000hours |
| Dimensions (W x D x H) | 166 x 137 x 68.5 mm |
| Weight | Product Weight: 1KG Packing Weight: 1.1KG |
| Working Environment | Working temperature: -40～80℃ ; Storage temperature: -40～80℃ Relative Humidity: 5%~95 %( no condensation) |
| Warranty | 1-year replacement with new item; 3-years for main parts. |

Management features:

| Interface | IEEE802.3X (Full-duplex), Port temperature protection setting Port green Ethernet Energy-saving setting Broadcast storm control based on port speed. The speed limit of the message flow in the access port. The minimum particle size is 64Kbps. |
| --- | --- |
| Layer 3 Features | L2+ network management, IPV4/IPV6 management L3 soft routing forwarding, Static route, Default route @ 128 pcs, APR@1024pcs |
| VLAN | 4K VLAN based on port, IEEE802.1q VLAN based on the protocol VLAN based on MAC Voice VLAN, QinQ configuration Port configuration of Access, Trunk, Hybrid |
| Port Aggregation | LACP, Static aggregation Max 8 aggregation groups and 8 ports per group. |
| Spanning Tree | STP (IEEE802.1d), RSTP (IEEE802.1w), MSTP (IEEE802.1s) |
| Industrial Ring Network Protocol | G.8032 (ERPS), Recovery time less than 20ms 250 Ring at most, Max 250 devices per ring. Multicast |
| Multicast | MLD Snooping v1/v2,Multicast VLAN IGMP Snooping v1/v2, Max 1024 multicast groups, Fast log out |
| Port Mirroring | Bidirectional data mirroring based on port |
| QoS | Flow-based Rate Limiting Flow-based Packet Filtering 8*Output queues of each port 802.1p/DSCP priority mapping Diff-Serv QoS, Priority Mark/Remark Queue Scheduling Algorithm (SP, WRR, SP+WRR) |
| ACL | Port-based Issuing ACL,ACL based on port and VLAN L2 to L4 packet filtering, matching first 80bytes message. Provide ACL based on MAC, Destination MAC address, IP Source, Destination IP, IP Protocol Type, TCP/UDP Port, TCP/UDP Port Range, and VLAN, etc. |
| Security | IP-MAC-VLAN-Port binding ARP inspection, Anti-DoS attack AAA & RADIUS, MAC learning limit Mac black holes, IP source protection IEEE802.1X & MAC address authentication Broadcast storm control,Backup for host datum SSH 2.0, SSL, Port isolation, ARP message speed limit User hierarchical management and password protection |
| DHCP | DHCP Client, DHCP Snooping, DHCP Server, DHCP Relay |
| Management | One-key recovery Cable Diagnose, LLDP Web Management (HTTPS) NTP, System work log,Ping Test CPU instant utilization status view Console/AUX Modem/Telnet/SSH2.0 CLI Download & Management on FTP, TFTP, X modem, SFTP, SNMP V1/V2C/V3 |
| System | Category 5 Ethernet network cable Web browser: Mozilla Firefox 2.5 or higher, Google browser chrome V42 or higher, Microsoft Internet Explorer10 or later; TCP/IP, network adapter, and network operating system (such as Microsoft Windows, Linux, or Mac OS X) installed on each computer in a network |

Dimensions

Ordering Information

| LBT128BGS-SFP | 8-port 10/100/100BASE-TX+12G SFP Managed Industrial Switch |
| --- | --- |