LBT224GS-M-SFP

24-port 10//100/1000BASE-TX+2G SFP Managed Industrial Switch

Interface

- 24-port 10/100/1000Base-Tx RJ-45 with auto negotiation and auto-MDI/ MDI-X function

- 2-port gigabit SFP

Industrial conformance

- 12V to 52V DC, redundant power input

- -40℃ to 80℃ operating temperature

- IP40 aluminum housing

- Supports 6KV DC Ethernet ESD protection

- Supports 6KV DC EFT protection for power line

- Free Fall, Shock and Vibration Stability

- DIN-rail design

LBT224GS-M-SFP an hardened 24-port Industrial Ethernet Switch, provides non-blocking wire-speed performance in harsh industrial environment. It also provides 24-port 10/100/1000BASE-TX RJ45 +2G SFP delivered in an IP40 rugged, strong case with redundant power system. Its slim case is suitable for such applications as surveillance system deployments, control management and wireless service in climatically demanding environments with wide temperature range from -40℃ to 80℃.

The LBT224GS-M-SFP provides the flexibility to all kinds of 10/100/1000Mbps Ethernet Media on RJ-45 port and performs highly stable fiber performance.

The LBT224GS-M-SFP is packaged in a compact IP40 case with DIN rail installation. It provides an integrated power supply with a wide range of voltages for worldwide operation. It also offers dual-redundant, reversible polarity 12V DC to 52V DC power supply inputs for high availability applications requiring dual or backup power inputs.

ROBOT

ROBOT

Specifications:

| Model No. | LBT224GS-M-SFP |
| --- | --- |
| 1000Base-TX Port | 24 port RJ-45 auto-MDI / MDI-X |
| 1000Base-FX Port | 2 SFP Slot |
| Performance Specification | Bandwidth: 56Gbps Packet Buffer Memory: 12Mbit Packet Forwarding Rate: 42Mpps MAC Address Table: 8K |
| Installation | DIN rail |
| Maximum Frame Size | 10240bytes packet size |
| Flow Control | Back pressure for half duplex, IEEE 802.3x pause frame for full duplex |
| Enclosure | IP40 Aluminum housing |
| LED Indicator | Power: Red Copper / Fiber: Link/Act Ethernet: Yellow |
| Power Input | 12 to 52V DC redundant power |
| Power Consumption | < 20W |
| Surge protection | ± 4KV |
| Network Protocols | IEEE802.3 10BASE-T; IEEE802.3i 10Base-T; IEEE802.3u;100Base-TX/FX; IEEE802.3ab 1000Base-T; IEEE802.3z 1000Base-X; IEEE802.3x; |
| Network cables | 10BASE-T: Cat3,4,5 UTP(≤100 meter) 100BASE-TX: Cat5 or later UTP(≤100meter) 1000BASE-TX: Cat6 or later UTP(≤100 meter) |
| Industry Standard | FCC CFR47 Part 15,EN55022/CISPR22, Class A IEC61000-4-2 (ESD): ±8kV (contact), ±12kV (air) IEC61000-4-3 (RS): 10V/m (80~1000MHz) IEC61000-4-4 (EFT): Power Port: ±4kV; Data Port: ±2kV IEC61000-4-5 (Surge): Power Port: ±2kV/DM, ±4kV/CM; Data Port: ±2kV IEC61000-4-6 (CS): 3V (10kHz-150kHz); 10V (150kHz-80MHz) IEC61000-4-16 (Common mode conduction): 30V (cont.), 300V (1s) |
| Certification | CE FCC Rohs compliance |
| MTBF | >300,000hours |
| Dimensions (W x D x H) | 166*137*68.5mm |
| Weight | Product Weight: 1.3KG Packing Weight: 1.4KG |
| Working Environment | Working temperature: -40～80℃ ; Storage temperature: -40～80℃ Relative Humidity: 5%~95 %( no condensation) |
| Warranty | 1-year replacement with new item; 3-years for main parts. |

Software:

| Port Features |  | IEEE 802.3x Flow control |  |  |
| --- | --- | --- | --- | --- |
|  |  | Interface counters |  |  |
|  |  | Storm Control |  |  |
| Link Aggregation |  | Static Aggregation |  |  |
|  |  | Dynamic Aggregation, LACP |  |  |
|  |  | Supports up to 8 aggregation groups, each group up to 8 ports Support source MAC, destination MAC, source destination MAC, source IP, destination IP, source destination IP routing strategy |  |  |
| MAC Table Management |  | Support |  |  |
| Forwarding mode |  | Support IVL forwarding mode |  |  |
| Static MAC Address |  | Support |  |  |
| MAC Address Binding |  | Support |  |  |
| MAC Address Filtering |  | Support |  |  |
| Mirror |  | Port-based Mirror |  |  |
| Spanning Tree |  | IEEE802.1d STP |  |  |
|  |  | IEEE802.1w RSTP |  |  |
|  |  | IEEE802.1s MSTP |  |  |
| ERPS (G.8032) |  | Single Ring, Sub-Ring, Multi-Ring |  |  |
|  |  | Load balance |  |  |
|  |  | Recovery Time < 50ms |  |  |
| GVRP |  | Supported |  |  |
| QINQ |  | VLAN-based QinQ |  |  |
| LLDP |  | LLDP |  |  |
|  |  | LLDP-MED |  |  |
| Loop Protection |  | Supported |  |  |
| VLAN |  | 4094 VLAN |  |  |
|  |  | Port-based VLAN |  |  |
|  |  | GVRP |  |  |
| Protocol VLAN |  | MAC-based VLAN |  |  |
|  |  | IP Subnet-based VLAN |  |  |
| Voice VLAN |  | Static entry |  |  |
|  |  | LLDP-MED |  |  |
| Port Isolate |  | Support |  |  |
| IGMP |  | Snooping IGMP V1 Snooping |  |  |
|  |  | IGMP V2 Snooping |  |  |
|  |  | IGMP V3 Snooping |  |  |
|  |  | Source Port Check |  |  |
|  |  | IGMP Fast Leave |  |  |
|  |  | IGMP Filter |  |  |
| MLD Snooping |  | MLD V1 Snooping |  |  |
|  |  | MLD V2 Snooping |  |  |
|  |  | Source Port Check |  |  |
|  |  | MLD Fast Leave |  |  |
|  |  | MLD Filter |  |  |
| Security |  |  |  |  |
| ACL |  | ACL (IP Standard ACL), IPv4/IPv6 |  |  |
|  |  | MAC extend ACL |  |  |
|  |  | IP extend ACL |  |  |
|  |  | L2, L3, L4 fields match |  |  |
|  |  | log, redirection, mirroring, speed limit |  |  |
| QoS |  | QoS Class, Remarking |  |  |
|  |  | SP, WRR queue scheduling |  |  |
|  |  | Ingress port-based rate-limit |  |  |
|  |  | Ingress priority-based rate-limit |  |  |
|  |  | Egress port-based rate-limit |  |  |
|  |  | Egress queue-based rate-limit |  |  |
|  |  | Policy-based QoS |  |  |
|  |  | Storm Control |  |  |
|  |  | 8 queues per port |  |  |
| Dot1x |  | Port Access Control |  |  |
|  |  | User Access Control |  |  |
|  |  | MAC-address Access Control |  |  |
| AAA |  | Support 802.1x |  |  |
|  |  | Support RADIUS |  |  |
|  |  | Supports authentication, authorization, and accounting through RADIUS server |  |  |
| Management |  |  |  |
| User management |  |  | Password protection |  |
| SNMP |  |  | SNMP V1/V2c/V3 |  |
| RMON |  |  | Statistics Group |  |
|  |  |  | History Group |  |
|  |  |  | Alarm Group |  |
|  |  |  | Events Group |  |
| Web management |  |  | HTTP V1.1, HTTPS |  |
| Firmware Upgrade |  |  | Reversion Upgrade |  |
| IPv4/IPv6 |  |  | Support |  |
| CLI |  |  | Support |  |
| Syslog/Debug |  |  | Support |  |
| Diagnosis |  |  | Ping (IPv4), ping (IPv6) |  |
|  |  |  | Trace route(IPv4) |  |
|  |  |  | Trace route(IPv6) |  |
| Management |  |  |  |  |  |
| NTP | NTP Client |  |  |
| DHCP | DHCPv4 Client, DHCPv6 Client |  |  |
| DHCP Snooping | DHCP snooping table |  |  |
|  | Trust Port |  |  |
| IPv6 DHCP-Guard | Support |  |  |
| Telnet | Telnet Server |  |  |
| TFTP | TFTP Client |  |  |
| SSH | SSH Server |  |  |

Ordering Information

| LBT224GS-M-SFP | 24-port 10/100/100BASE-TX+2G SFP Managed Industrial Switch |
| --- | --- |