LBT824GST4-SFP

24-Port gigabit RJ45 +8G SFP combo+4-Port 10G SFP

L3 Managed Industrial Switch

Features

- Data control: support 802.3x full duplex flow control, support network storm suppression

- Redundant network: support STP/RSTP/MSTP, support ERPS (self-healing time <20ms)

- Support IPv6, static routing, Dynamic routing (RIP,OSPF)

- Support RIPng, OSPFv3, DHCP Server, DHCP replay, Qos

- Multicast management: Support IGMP Snooping V1/V2/V3

- VLAN: support IEEE 802.1q VLAN, effectively isolated broadcast domain

- Link aggregation: support link static/dynamic aggregation, provide perfect bandwidth utilization

- QoS: Support COS\DSCP, 4 queues, support WRR\SP scheduling mode

- Security management: support ACL access control list, support 802.1x, support user classification management

- Management function: support Web, SNMP management mode

- Monitoring maintenance: support port mirroring, interface status monitoring, log management

- Alarm function: support power supply, port, abnormal state relay output

Rack mounted Aluminum Industrial Switch

LBT824GST4-SFP switch is a high performance 10G SFP+ L3 core switch. It provides Static routing and dynamic routing with advanced hardware processing capacity. The machine adopts modular design, which supports up to 4 Port 10G SFP+, and retains the expansion ability to further support 10GE when realizing gigabit convergence or access. It supports IPv4/IPv6 hardware double stack and line speed forwarding, so that customers can easily cope with the coming IPv6 era, which provides high capacity exchange capacity. LBT824GST4-SFP switch has the operating temperature of -40℃ ~ 85℃, has the super firmness to adapt to all kinds of harsh environment, can also be very convenient to install in the machine room cabinet. The rack-mounted characteristics and wide temperature operation make the switch a plug and play industrial-grade device, providing a reliable and convenient solution for users to network their Ethernet devices.

Strong processing ability

LBT824GST4-SFP switch is based on the powerful and stable

processing power of VCore-III MIPS-based CPU

LBT824GST4-SFP provides up to 1.2Tbps of switching capacity

port full speed filtering and forwarding 406Mpps

To ensure that the trunk network of large traffic data without blocking

Forwarding.

ROBOT

ROBOT

Specifications:

| Model No. | LBT824GST4-SFP |
| --- | --- |
| Interface | 24-Port 10/100/1000Mbps RJ45 8-Port 10/100/1000Mbps SFP(combo) 4-Port 10G SFP+ 1-Port console |
| Performance Specification | Bandwidth(switch capacity): 1.2Tbps Packet Buffer Memory:32Mbit Packet Forwarding Rate: 406Mpps MAC Address Table: 32K |
| Installation | Bracket mounted |
| Maximum Frame Size | 16360bytes |
| Flow Control | Back pressure for half duplex, IEEE 802.3x pause frame for full duplex |
| Enclosure | IP50 Aluminum housing |
| LED Indicator | Power: Red Fiber: Link 2(Green) Ethernet: Yellow |
| Dual Power Input | 36 to 72V DC &110～240V/50～60Hz AC |
| Power Consumption | < 30watts |
| Surge protection | ±4KV |
| Network Protocols | IEEE802.3 10BASE-T; IEEE802.3i 10Base-T; IEEE802.3u;100Base-TX/FX; IEEE802.3ab 1000Base-T; IEEE802.3z 1000Base-X; IEEE802.3x; |
| Network cables | 10BASE-T: Cat3,4,5 UTP(≤100 meter) 100BASE-TX: Cat5 or later UTP(≤100meter) 1000BASE-TX: Cat6 or later UTP(≤100 meter) |
| Industry Standard (EMC 4) | FCC CFR47 Part 15,EN55032 IEC61000-4-2 (ESD): ±8kV (contact), ±12kV (air) IEC61000-4-3 (RS): 10V/m (80~1000MHz) IEC61000-4-4 (EFT): Power Port: ±4kV; Data Port: ±2kV IEC61000-4-5 (Surge): Power Port: ±2kV/DM, ±4kV/CM; Data Port: ±2kV IEC61000-4-6 (CS): 3V (10kHz-150kHz); 10V (150kHz-80MHz) IEC61000-4-16 (Common mode conduction): 30V (cont.), 300V (1s) |
| MTBF | 300,000hours |
| Dimensions (W x D x H) | 442×194×44mm |
| Weight | Product Weight: 4KGS Packing Weight: 4.6KGS |
| Working Environment | Working temperature: -40～85℃ ; Storage temperature: -40～85℃ Relative Humidity: 5%~95 %( no condensation) |

Management features:

| Interface | IEEE802.3X (Full-duplex), Port temperature protection setting Port green Ethernet Energy-saving setting Broadcast storm control based on port speed. The speed limit of the message flow in the access port. The minimum particle size is 64Kbps. |
| --- | --- |
| Layer 3 Features | L3 network management, IPV4/IPV6 management L3 soft routing forwarding, Static route, Default route @ 128 pcs, APR@1024pcs |
| VLAN | 4K VLAN based on port, IEEE802.1q VLAN based on the protocol VLAN based on MAC VLAN based on IP Voice VLAN, QinQ configuration Port configuration of Access, Trunk, Hybrid |
| Port Aggregation | LACP, Static aggregation Max 14 aggregation groups and 16 ports per group. |
| Spanning Tree | STP (IEEE802.1d), RSTP (IEEE802.1w), MSTP (IEEE802.1s) |
| Industrial Ring Network Protocol | G.8032 (ERPS), Recovery time less than 20ms 250 Ring at most, Max 250 devices per ring. Multicast |
| Multicast | MLD Snooping v1/v2,Multicast VLAN Registration (MVR) IGMP Snooping v1/v2, Max 1024 multicast groups, Fast log out |
| Port Mirroring | Bidirectional data mirroring based on port |
| QoS | SP（Strict Priorty） WFQ (Weighted Fair Queuing) WRR (Weighted Round Robin) Random Early Detect (RED) WeightedRandomEarlyDetection (WRED) Head Of Line Best-Effort Service First come first served 802.1p Cos/Tos，QOS Differentiated Service |
| ACL | Port-based Issuing ACL,ACL based on port and VLAN L2 to L4 packet filtering, matching first 80bytes message. Provide ACL based on MAC, Destination MAC address, IP Source, Destination IP, IP Protocol Type, TCP/UDP Port, TCP/UDP Port Range, and VLAN, etc. |
| Security | Support IP ACL，MAC ACL，Vlan ACL,support ACL based on L3 L4 IP+MAC Port binding Support IP Source Guard, RP Detection Support IEEE 802.1x port verification, support Radius |
| DHCP | DHCP Client, DHCP Snooping, DHCP Server, DHCP Relay |
| IP routing | Supports port-based input/output bandwidth management. Supports static routes. IPV6 Supports the virtual route redundancy protocol VRRP. Dynamic routes such as RIP, RIPng, OSPF, and OSPFv3 are supported. |
| System maintenance | Supports configuration file uploading and downloading. Upload the upgrade package. Supports factory restore on WEB, Supports the system log function. Support Ping 、tracert、cable detection and other diagnostic functions. Supports WEB management (HTTP and HTTPS). Supports CLI management. Supports Telnet remote management. Supports SSH 2.0. Supports SNMP network management. Support RMON; Supports NTP clients. |