300Mbps Outdoor Wireless Access Point

Model: T300-AP20Q

Description:

T300-AP20Q is an Qualcomm Chipset Outdoor wireless AP/CPE. It comply with IEEE 802.11b/g/n standard, 300Mbps high speed, with N type antenna connector, 800mW high power for wider outdoor wifi range.

It support wireless AP, Gateway, WiFi Repeater, Bridge, WISP, WDS operation mode, then with N type connector for detachable omni/panel antenna, user can flexibly choose a suitable wireless mode to build or extend the wireless networking based on exist networking structure.

T300-AP20Q with metal waterproof, dustproof and sunscreen shell, temperature adaptive and board protective to work in harsh outdoor environment, meanwhile, the auxiliary cooling function make T300-AP20Q work normal at -20℃ to 55℃.

T300-AP20Q support special frequency from 2.312GHz to 2.4835GHz, total 27 channels to avoid the WiFi Interference, to guarantee the stable wireless connection.

Meantime, it support Power over Ethernet, support passive PoE, easy in setup even no power socket nearby.

Last and important, T300-AP20Q work with Yuncore’s AC controller for central management under wireless AP operation mode, then can work with cloud server for captive portal authentication or advertisement when AC controller or T300-AP20Q under gateway operation mode..

| Specification: |  |
| --- | --- |
| Hardware: |  |
| Chipset | Qualcomm QCA9531 |
| Wireless | 802.11N, 2T2R 300M MIMO Technology |
| Memory | 64MB DDR2 RAM (128MB available by extra cost) |
| Flash | 16MB |
| Interface | 1* 10/100 Mbps RJ45 Port, support 12~24V PoE |
|  | 1 * Reset Button |
| Antenna | SMA Connector, can connect with panel antenna/omni antenna |
| LED Light | Power, WiFi |
| Size | L: 153.2mm, W: 79.5mm, H: 234.5 mm |
| Power | POE 24V/1A (48V PoE is optional);110V/220V |
| RF Date: |  |
| RF Data | 802.11b/g/n: |
|  | 2.312GHz to 2.4835GHz |
|  | Korea, Japan, ETSI, FCC, Debug |
| Modulation | OFDM = BPSK,QPSK, 16-QAM, 64-QAM |
|  | DSSS = DBPSK, DQPSK, CCK |
| Data Rate | 300 Mbps |
| Receive Sensitivity | 802.11n: -70dbm@MCS7, -88dbm@MCS0. 802.11g: -72dbm@54Mbps, -88dbm@6Mbps. 802.11b: -85dbm@11Mbps, -94dbm@1Mbps. |
| RF Output Power | 802.11n: @MCS7:25±2DB, @MCS0:27±2DB. 802.11g: @54M:26±2DB, @6M:28±2DB. 802.11b: @11M:27±2DB, @1M:29±2DB |
| EVM | 802.11n: ≤-28 DB 802.11g: ≤-25 DB 802.11b: ≤-10 DB |
| PPM | ±20ppm |

| Firmware |  |
| --- | --- |
| Operation Mode | Gateway/AP/WiFi Repeater/WISP/WDS |
| Protocol/Standard | IEEE 802.3(Ethernet) |
|  | IEEE 802.3u(Fast Ethernet) |
|  | IEEE 802.11b/g/n(2.4G WLAN) |
| Wireless | Auto-Channel selection |
|  | Distance Control (802.1x Ack timeout) |
|  | Multiple SSID (4 SSID) |
|  | BSSID |
| Security | WEP Encryption-64/128/152 bit |
|  | WPA/WPA2 Personal (WPA-PSK using TKIP or AES) |
|  | WPA/WPA2 Enterprise (WPA-EAP using TKIP) |
|  | Hide SSID |

| System Setting | Web-based configuration (HTTP/Telnet) |
| --- | --- |
| Firmware upgrade | Upgrade firmware via web browser or TFTP |
| Administration | Admin Password can be configured |
| System monitoring | Status in hand , useful statistic and Event log |
| Log | Supports local logs, logs host, logs file export |
| Reset | Reset or factory defaults |
| Backup | Restore settings & configuration of the device to local file |
| Physical Property: |  |
| Temperature | Working: -20°C~55°C |
|  | Storage: -40°C to 70°C |
| Humidity | 5%~95% (typical) |
| Package: |  |
| T300-AP20Q unit |  |
| PoE Adapter |  |
| User Manual |  |
| LAN Cable |  |

Remark:  antenna is optional by extra cost.

Accessory Pictures:

T300-AP20Q Application:

- Work with omni antenna for wifi coverage:                2. Work with Panel antenna for wifi transmit