11ax 3000Mbps Outdoor Wireless AP-Omni Antenna

Model:T300-HQ830-P2

Description:

T300-HQ830-P2 is a high-performance 4T4R dual-band 802.11AX product that can provide 2.4G and 5.8GHz dual-band wireless services. 2.4G provides a maximum access rate of 574Mbps, and 5G provides a maximum access rate of 2402Mbps.

Main Features:

- Has better transmission quality, higher transmission rate, and concurrent rate up to 3000Mbps.

- Using IPQ5018 chip, it greatly expands the user capacity and can support 128 users.

- The heat sink adopts snap-on structure design and surface nano-coating treatment, which makes the heat dissipation effect more ideal.

- The radio frequency uses high-power FEM, which has stable performance and farther coverage.

- The addition of lightning protection board components greatly enhances the protection capabilities of the equipment;

Hardware Specification:

| Model | T300-HQ830-P2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CPU | IPQ5018 + QCN6102 |  |  |  |  |
| Flash | SPI NOR 8MB SPI NAND 128MB |  |  |  |  |
| DDR3 | 512MB*1 |  |  |  |  |
| 2.4G working frequency | 2.4GHz ~ 2.484GHz |  |  |  |  |
| 2.4G WIFI protocol | 802.11 b / g / n / ac / ax |  |  |  |  |
| 5G working frequency | 5.150GHz～5.825GHz |  |  |  |  |
| 5G WIFI protocol | 802.11 a / n / ac / ax |  |  |  |  |
| Internal 2.4G | 2*2.4G: 5dBi |  |  |  |  |
| Internal 5G | 2*5.8G: 7dBi |  |  |  |  |
| Max rate | 2.4G: 574Mbps; 5G: 2402Mbps |  |  |  |  |
| 2.4G TX Power | 802.11b | 11M | 24±2dBm | 1M | 24±2dBm |
|  | 802.11g | 54M | 21±2dBm | 6M | 23±2dBm |
|  | 802.11n HT20 | MCS7 | 20±2dBm | MCS0 | 22±2dBm |
|  | 802.11n HT40 | MCS7 | 20±2dBm | MCS0 | 22±2dBm |
|  | 802.11ax HE20 | MCS11 | 18±2dBm | MCS0 | 22±2dBm |
|  | 802.11ax HE40 | MCS11 | 18±2dBm | MCS0 | 22±2dBm |
| 5G TX Power | 802.11a | 54M | 19±2dBm | 6M | 22±2dBm |
|  | 802.11n HT20 | MCS7 | 18±2dBm | MCS0 | 21±2dBm |
|  | 802.11n HT40 | MCS7 | 18±2dBm | MCS0 | 21±2dBm |
|  | 802.11ac VHT20 | MCS9 | 17±2dBm | MCS0 | 21±2dBm |
|  | 802.11ac VHT40 | MCS9 | 17±2dBm | MCS0 | 21±2dBm |
|  | 802.11ac VHT80 | MCS9 | 17±2dBm | MCS0 | 21±2dBm |
|  | 802.11ax HE20 | MCS11 | 17±2dBm | MCS0 | 21±2dBm |
|  | 802.11ax HE40 | MCS11 | 17±2dBm | MCS0 | 21±2dBm |
|  | 802.11ax HE80 | MCS11 | 17±2dBm | MCS0 | 21±2dBm |
|  | 802.11ax HE160 | MCS11 | 17±2dBm | MCS0 | 21±2dBm |
| 2.4G Receiving Sensitivity | 802.11b | 11M | -85dBm | 1M | -90dBm |
|  | 802.11g | 54M | -72dBm | 6M | -90dBm |
|  | 802.11n HT20 | MCS7 | -70dBm | MCS0 | -87dBm |
|  | 802.11n HT40 | MCS7 | -68dBm | MCS0 | -85dBm |
|  | 802.11ax HE20 | MCS11 | -60dBm | MCS0 | -83dBm |
|  | 802.11ax HE40 | MCS11 | -56dBm | MCS0 | -83dBm |
| 5G Receiving Sensitivity | 802.11a | 54M | -72dBm | 6M | -92dBm |
|  | 802.11n HT20 | MCS7 | -70dBm | MCS0 | -90dBm |
|  | 802.11n HT40 | MCS7 | -68dBm | MCS0 | -88dBm |
|  | 802.11ac VHT20 | MCS8 | -70dBm | MCS0 | -88dBm |
|  | 802.11ac VHT40 | MCS9 | -68dBm | MCS0 | -88dBm |
|  | 802.11ac VHT80 | MCS9 | -58dBm | MCS0 | -85dBm |
|  | 802.11ax HE20 | MCS11 | -62dBm | MCS0 | -88dBm |
|  | 802.11ax HE40 | MCS11 | -58dBm | MCS0 | -86dBm |
|  | 802.11ax HE80 | MCS11 | -55dBm | MCS0 | -84dBm |
|  | 802.11ax HE160 | MCS11 | -51dBm | MCS0 | -82dBm |
| 2.4G EVM | 802.11b:≤-10 dB ；802.11g: ≤-25 dB ；802.11n:≤-28 dB；802.11ax:≤-35 dB |  |  |  |  |
| 5G EVM | 802.11a:≤-25 dB ；802.11n: ≤-28 dB ；802.11ac:≤-32 dB；802.11ax:≤-35 dB |  |  |  |  |
| ppm | ±20ppm |  |  |  |  |
| WAN | 1*10/100/1000M support 48V POE |  |  |  |  |
| LAN | 1*Console port |  |  |  |  |
| Reset | Reset to factory settings by pressing 6-10 seconds |  |  |  |  |
| Indicators | PWR, SYS, WAN, 2.4G, 5.8G |  |  |  |  |
| Power | PoE 802.3at |  |  |  |  |
| Max Power Consumption | <16W |  |  |  |  |
| Dimension | 304mm × 181mm × 88mm |  |  |  |  |
| Weight | 1.6kg |  |  |  |  |
| Temperature | Working: -20°C to 55°C；Storage: -40°C to 70°C；Humidity: 5％～95％(Non-Condensing) |  |  |  |  |
| IP Grade | IP67 |  |  |  |  |
| ESD | Air:±8K, Touch:±4K |  |  |  |  |
| Electrical Surge | Common Mode: 4K, Differential Mode: 2K |  |  |  |  |

Firmware

| Home Page | Working mode: FIT/ FAT mode |
| --- | --- |
|  | Wireless AP download rate, wireless AP upload rate, runtime |
|  | Device information, device description, intranet information, WIFI information |
| Working Mode | Gateway mode: The device connects to the wide area network through a WAN port and then uses static IP, DHCP, or PPPOE to connect to the internet |
|  | AP mode: The device covers other clients and devices with wireless network signals, and connects to other routers through Ethernet to access the wide area network |
| Wireless feature | SSID broadcasting |
|  | SSID quantity: 4 (2.4GHz)+4()5GHz) |
|  | Hide SSID |
|  | Wireless encryption: OPEN、WPA/WPA2PSK-TKIPAES、WPA3PSK-TKIPAES |
|  | MAC filter |
|  | Band steering (5G WiFi priority) |
|  | WiFi timed shutdown |
|  | User isolation: wireless network isolation, AP internal isolation |
|  | TX power adjustable |
|  | Wireless client quantity limit |
|  | QoS: WMM |
| Network feature Cloud Platform Server | Connection method: static address, obtained from management server, obtained from gateway |
|  | DHCP server: It can be either a built-in DHCP server or an external DHCP server |
|  | Cloud Platform Server support |
|  | Firewall |
|  | PPPOE |
| Device management | Backup |
|  | Recovery |
|  | Reset to factory |
|  | Timed reboot/ Instant reboot |
|  | Firmware upgrade |
|  | System log |
|  | FIT/ FAT AP switch |

Interface

Dimension

Accessory

| AP | 1 |
| --- | --- |
| Mounting Accessories | 2 |
| Gift Box | 1 |